ClimateIQ Smart AC Controller — Full User Manual

Read me first. This manual is written in plain language and assumes you have

never used a device like this before. Take it one step at a time. If something

doesn't work, jump to Section 14 — Troubleshooting at the end. Nothing you do

with the buttons or the dashboard can permanently break the device.

Table of contents

1. What this device is (in one paragraph)

2. Simple words you need to know (glossary)

3. The parts of the device

4. The 5‑minute quick start

5. First‑time setup (the long, careful version)

6. The status light — what every colour means and what to do

7. The button — the only two things it does

8. Teaching the device your AC remote (very important)

9. Setting up the radar (presence sensor)

10. How the device decides to turn the AC on and off

11. Everyday use (the phone app / cloud)

12. Saving energy automatically

13. Updating the device software (firmware)

14. Troubleshooting (when something looks wrong)

15. Frequently asked questions

16. Do's and don'ts

17. One‑page quick reference (print this)

1. What this device is (in one paragraph)

ClimateIQ is a small box that sits near your air conditioner (AC) and makes it

"smart." It sends the same invisible infrared signals your AC remote sends, so

it can turn the AC on and off and change the temperature by itself. It can **tell

when someone is in the room using a radar sensor, it knows the room temperature

and humidity, and it follows a daily schedule** you set from a phone app. When

the room is empty it saves electricity by raising the temperature and then

switching the AC off. It connects to the internet over WiFi (or a SIM/cellular

connection) so you can control it from anywhere.

2. Simple words you need to know (glossary)

AC — your air conditioner.

IR / Infrared — the invisible light your AC remote uses. The device uses the

same thing. This is why the device must be able to "see" the AC.

Radar / presence sensor — the part that senses if a person is in the room.

It does not use a camera and does not record anything. It just senses

movement and a person's presence.

WiFi — your home/office wireless internet.

GSM / Cellular — internet using a SIM card, for places with no WiFi.

Setup mode (hotspot / AP mode) — a special mode where the device makes its

own little WiFi network so you can connect your phone to it and set it up.

Dashboard — the setup web page you open in a browser.

Schedule — a daily plan (set in the app) that says when the AC should be on

and at what temperature.

Eco — an energy‑saving mode: the AC keeps running but at a warmer temperature.

Firmware — the software inside the device. It can be updated.

Cloud / app / backend — the phone app or online service you use day to day.

3. The parts of the device

Status light (RGB LED) — one small light that changes colour to tell you what

the device is doing. This is your main way of knowing it's healthy. See Section 6.

Presence light — a separate small light that turns **on only when the radar is

active and a person is in the room**. If it's off, either the room is empty or

radar automation is turned off. This is normal.

The button — one physical button. It only does two things (Section 7).

Infrared blaster — the part that "speaks" to your AC. It must point toward

the AC with nothing blocking it.

Radar sensor — senses people. Should face into the room, not the doorway/corridor.

You don't need to open the device or touch any wiring. Everything in this manual is done with the button, your phone, or the app.

4. The 5‑minute quick start

If you just want it working fast, do these steps. Detailed versions follow later.

1. Plug in / power on the device. Wait about 30 seconds.

2. The light blinks cyan (greenish‑blue) = it wants to be set up. (If it doesn't,

press the button once to force setup mode.)

3. On your phone, open WiFi settings and connect to:

Network name: ECO_<device code>

Password: password123

Note: the setup network name is unique to each device — it starts with ECO_ followed by your device’s code, for example ECO_C0CDD6FB644C.

4. Open a web browser and type: `192.168.4.1` then press Go.

5. When asked to log in: Username admin, Password admin.

6. Go to Network Settings, pick WiFi, type your home WiFi name and password,

press Save & Reboot.

7. Wait. The light goes blue (connecting) → amber/yellow (almost there) →

green heartbeat (a brief green flash every 5 seconds = fully working). ✅

8. Teach it your AC remote (Section 8) and you're done.

5. First‑time setup (the long, careful version)

Step 5.1 — Power on

Connect the device to power. Give it about half a minute to start. While starting,

the light may be solid white.

Step 5.2 — Get into Setup mode

If the device has never been connected to WiFi, it goes into Setup mode by

itself. You'll see the light blinking cyan (a greenish‑blue).

If it doesn't, press the button once (a quick tap). That forces Setup mode.

Step 5.3 — Connect your phone to the device

On your phone or laptop, open the WiFi list. You will see a network called

`ECO_<device code>`. Connect to it using the password `password123`.

Tip: each device’s network has its own name beginning with ECO_ (for example ECO_C0CDD6FB644C), so if you have more than one device you can tell them apart.

Your phone may warn "No Internet" on this network. That's expected. The device's own network has no internet — you're only using it to set things up. Stay connected.

Step 5.4 — Open the dashboard

Open any web browser (Chrome, Safari, etc.). In the address bar type exactly:

192.168.4.1

Press Go / Enter. A login box appears.

Username: admin

Password: admin

Step 5.5 — Enter your WiFi

Find Network Settings on the page.

Choose WiFi mode.

Type your real WiFi name (SSID) exactly, including capital letters.

Type your real WiFi password exactly.

Press Save & Reboot.

No WiFi at this location? Choose GSM / Cellular instead (needs a working SIM in the device). Note: software updates over the air work on WiFi only, not GSM.

Step 5.6 — Watch it connect

The device restarts and joins your WiFi. Follow the light:

Blue blinking = connecting to WiFi.

Amber/yellow blinking = on WiFi, finishing the connection to the cloud.

Green flash every 5 seconds = fully connected and healthy. ✅

If it gets stuck on amber, see Section 14.

Step 5.7 — Teach the AC remote and set the schedule

Do Section 8 (teach the remote) now, then set your schedule in the app (Section 11).

6. The status light — what every colour means and what to do

This one light tells you almost everything. Here is the complete list.

Steady states (the device is in this state until something changes)

| Light | How it looks | What it means | What you should do |
| --- | --- | --- | --- |
| White | Solid (not blinking) | Starting up / booting | Wait ~30 seconds |
| Cyan (greenish‑blue) | Blinking | Setup mode — making its own hotspot | Connect to ECO_<device code> to set it up |
| Blue | Blinking | Connecting / reconnecting to WiFi | Wait. If stuck, check WiFi name/password |
| Green | One brief flash every 5 seconds (heartbeat) | Everything is healthy — online and sending data | Nothing. This is the good one ✅ |
| Amber / Yellow | Blinking | On WiFi but not yet talking to the cloud | Wait; check your internet. It keeps retrying |
| Magenta (pink/purple) | Blinking | Cellular (GSM) is connecting | Wait |
| Magenta (pink/purple) | Brief flash every 5 seconds (heartbeat) | Cellular connected and healthy | Nothing. Good on GSM ✅ |
| Magenta (pink/purple) | Solid, not blinking | Offline backup mode — no internet, running the room locally on radar only | Nothing required. It's a safe fallback and fixes itself when internet returns |
| Red | Fast blinking | Error | See Troubleshooting. Try powering off and on |

Quick one‑time flashes (an event just happened, then it returns to normal)

| Flash | Meaning |
| --- | --- |
| White, quick single flash | The device just sent a signal to your AC |
| Green, twice | Something worked (settings saved, button learned) |
| Red, three times | Something failed (try again) |
| Yellow, three times | A warning |
| Red/Blue fast alternating strobe (like police lights) | Learning failed — it didn't catch your remote's signal. Try again, closer |

The single most useful thing to remember: a calm green flash every 5 seconds means the device is online and working. Solid magenta means no internet but the room is still being handled locally — also OK.

7. The button — the only two things it does

There is one button. It does exactly two things:

| You do this | This happens |
| --- | --- |
| Tap it once (short press) | Enters Setup mode (makes the ECO_<device code> hotspot so you can change settings) |
| Hold it ~5 seconds then let go (long press) | Exits Setup mode and reconnects to your normal network |

That's it. You can't break anything with the button.

8. Teaching the device your AC remote (very important)

The device must learn your specific AC's signals, because every AC brand is

different. Do this in the dashboard (Setup mode → 192.168.4.1 → admin/admin),

in the section called IR Learning Center.

How learning works (read this once)

For each item below, you press the button on the web page first, then **point

your real AC remote at the device and press the matching remote button within about

10 seconds.** A green flash / success means it saved. A failure (or the red/blue

police strobe) means it didn't hear the remote — try again, closer, aimed straight

at the device.

What to teach (in order)

| On the web page press… | Then on your AC remote press… | What it saves |
| --- | --- | --- |
| Detect AC Protocol | Any button on the remote | Your AC's brand/"language" |
| Learn ON | The power‑ON button | The exact "turn on" signal |
| Learn OFF | The power‑OFF button | The exact "turn off" signal |
| Learn 24 °C | Set remote to 24° and press | The 24° signal |
| Learn 26 °C | Set remote to 26° and press | The 26° signal |
| Learn 28 °C | Set remote to 28° and press | The 28° signal |
| Learn 30 °C | Set remote to 30° and press | The 30° signal |

Good to know

You don't have to learn every temperature. If a temperature isn't learned, the

device builds a universal signal from the detected protocol. Learning them just

makes it more reliable.

Always detect the protocol first, then learn ON/OFF, then the temperatures.

If the AC ignores the device later, come back here and re‑learn (Section 14).

9. Setting up the radar (presence sensor)

The radar is what makes the AC react to people. It's in the dashboard, partly under

Developer Mode (advanced tools, password dev1234).

Auto‑Calibrate (teach it the empty room)

1. Make the room empty — everyone out, including you if you can, or stand very

still out of the radar's view.

2. Press Auto‑Calibrate Radar.

3. Wait up to about 2 minutes. Presence detection is paused while it learns the

empty room's background.

4. A success message means it's done.

Do this once after installing, and again if the radar gives false alarms.

Detection Range (how far it looks)

In Developer Mode → Radar Detection Range, set how far the radar should sense

(roughly 0.3 m to 11 m; there are quick 2 m / 4 m / 6 m buttons). **Set it just

big enough to cover your room** so it doesn't sense people in the corridor or the

next room.

Radar Presence Hold (how long stillness still counts as “occupied”)

In Developer Mode → Radar Presence Hold, set how many seconds the device keeps treating the room as occupied after the last movement it senses. If people in your room sit very still (reading, working at a desk), raise this so the AC doesn’t drop to Eco or switch off during brief stillness. Lower it to react faster once the room truly empties.

Factory Reset Radar

Restores the radar's own default settings if it's misbehaving. Re‑calibrate after.

Good to know: in Developer Mode the Automation status shows “Schedule active” during normal operation, or “Manual OFF hold — paused” when the AC has been turned off from the app and is being held off.

10. How the device decides to turn the AC on and off

This section explains the "brain." Read it so the AC's behaviour never surprises you.

Rule 1 — The schedule is the boss ("schedule is king")

The daily schedule (set in the app) says when the AC may run and at what

temperature.

Inside a scheduled time block → the AC is allowed to run.

Outside all blocks → the AC is forced off.

The radar only works inside scheduled hours. (If you have no schedule at

all, the radar runs freely all the time.)

Rule 2 — Radar saves energy in three steps

When radar automation is on and you're inside a scheduled block:

1. Person in the room → AC runs at the Normal temperature.

2. Room empty for the "Eco Delay" (e.g. 2 minutes) → AC raises to the Eco

temperature to save power.

3. Room still empty for the "Off Delay" (e.g. 5 minutes) → AC turns off.

4. Someone comes back → AC immediately returns to the Normal temperature.

Rule 3 — Manual remote presses

The device is always listening to your AC remote:

If you use the remote to change temperature or turn the AC on/off in a way that

disagrees with the schedule, the device will put it back to what the

schedule wants (the schedule wins).

If you press other remote buttons (fan speed, swing, mode), the device leaves

them alone — your adjustment stays.

This only applies to a remote using the same AC the device learned. Other

remotes are ignored (but shown in "Remote Activity" on the dashboard).

Rule 4 — Power on/off from the app

App “Turn OFF” → The AC turns off and stays off. It ignores the schedule and the radar — it will not come back on at the next scheduled block, and movement won’t wake it.

It stays off until you turn it back on from the app (press Turn ON, or set a temperature). This makes sure a deliberate “off” is always respected.

App “Turn ON” (or setting a temperature) → Releases the hold and resumes normal operation. Turning on only takes effect during scheduled hours;

outside those hours the schedule switches it back off shortly after.

Rule 5 — Flap delay (stops on/off flickering)

Right after the AC turns off, its moving air can look like "motion" to the radar. The

Flap Delay (a few seconds) makes the radar ignore this so the AC doesn't bounce

straight back on.

Rule 6 — Re‑confirm every few minutes ("enforce")

Every few minutes the device re‑sends the AC's intended state. This fixes any signal

the AC missed and keeps reality matching the schedule.

Rule 7 — Offline backup

If the internet is gone for about a minute and the clock isn't set, the device

switches to local‑only mode: it runs the room on radar and shows solid magenta.

It returns to normal automatically when the internet/time comes back.

Rule 8 — After a power cut

When power returns, the device reconnects, gets the time, and immediately applies

whatever the schedule says for right now (it doesn't wait for midnight). Inside a

block → AC comes on; otherwise it stays off.

11. Everyday use (the phone app / cloud)

Once set up, you mostly use the app. From there you can, in real time:

Set the temperature.

Turn the AC on or off (remember Rule 4 above about scheduled hours).

Turn radar automation on or off.

Set the Eco values — Eco temperature, Eco delay, Off delay.

Create or edit the daily schedule — time blocks for each weekday, each with its

own temperature and optional radar/eco settings. A change for today takes effect

right away.

Start a firmware update (Section 13).

When you send a command, the device does it, saves the setting, and sends back a

confirmation so the app can show it worked.

12. Saving energy automatically

You don't have to do anything special — it's automatic when radar automation is on:

Empty room for a while → temperature is raised (Eco) → then the AC turns off.

Someone returns → AC comes back to your normal comfort temperature.

You can tune how aggressive this is by changing Eco Delay (how soon it goes to

Eco) and Off Delay (how soon it switches off) in the app or Developer Mode.

Default values: Normal 24 °C · Eco 26 °C · Eco delay 2 min · Off

delay 5 min · Flap delay 10 s.

13. Updating the device software (firmware)

There are two ways to update, and a built‑in safety net.

Two ways:

From the cloud/app: the backend sends an update with a download link; the device

downloads, installs, restarts, and reports the result.

From the dashboard: Developer/Admin → OTA Firmware Update → upload a .bin

file from your computer.

Safety net (automatic rollback): a new firmware boots "on trial." It only becomes

permanent after the device proves it's healthy (reconnects to the cloud). If a bad

update keeps crashing, the device **automatically goes back to the previous working

version**. A bad update will not "kill" the device.

Notes:

Over‑the‑air updates need WiFi (not available on GSM).

The device won't reinstall the same version it's already running unless the update is

sent with "force."

14. Troubleshooting (when something looks wrong)

| What you see | What it means / what to do |
| --- | --- |
| Light blinks amber/yellow and won't go green | WiFi is fine but the internet/cloud isn't reachable. Check your internet. The device keeps retrying on its own. |
| Light is solid magenta | Offline backup mode — no internet. This is safe; the room still works on radar. It recovers by itself when the internet returns. |
| Light keeps blinking blue | Can't join WiFi. The WiFi name or password is probably wrong. Tap the button to enter Setup mode and re‑enter them. |
| AC won't respond at all | The AC remote may not be learned, or the device can't "see" the AC. Re‑do IR Learning Center (Protocol → ON/OFF → temperatures) and make sure nothing blocks the line of sight to the AC. |
| AC turns on/off "by itself" | That's the schedule + radar working. Check your schedule and radar setting in the app; adjust Eco/Off delays if needed. |
| Radar reacts to the corridor / next room | Reduce the Detection Range in Developer Mode and run Auto‑Calibrate with the room empty. |
| AC flicks back on right after turning off | Increase the Flap Delay so the radar is ignored a little longer after an OFF. |
| My temperature change keeps resetting | Inside scheduled hours the schedule wins. Change the temperature in the schedule/app, not on the handheld remote. |
| Swing/fan setting reverts | The periodic re‑confirm re‑sends the AC state; swing/fan may revert. The device manages power and temperature, not fan/swing. |
| Red/blue police‑light strobe during learning | It didn't catch the remote. Aim the remote straight at the device, get closer, and try again. |
| Can't open `192.168.4.1` | Make sure your phone is connected to ECO_<device code> (device must be in Setup mode = blinking cyan). Type the address without https. |
| Forgot WiFi / moved to a new place | Tap the button to enter Setup mode, reconnect to ECO_<device code>, and enter the new WiFi. Or use Wipe WiFi Credentials in System Admin. |
| Want to start fresh | System Admin → Reset IR Memory (forgets learned buttons) and/or Wipe WiFi Credentials (forgets WiFi). |
| Nothing works / totally stuck | Power the device off, wait 10 seconds, power it on. If still stuck, enter Setup mode and check your settings. |
| AC stays off and the schedule/radar won’t turn it on | Someone turned it OFF in the app. A manual OFF holds the AC off until a manual ON — press Turn ON (or set a temperature) in the app to release it. |

15. Frequently asked questions

Does the radar use a camera or record me?

No. It senses movement/presence only. There is no camera and no recording.

Will it work if my internet goes down?

Yes. It switches to offline backup mode (solid magenta) and keeps running the room on

radar. It reconnects automatically.

Do I have to learn every temperature button?

No. Learning ON, OFF, and the protocol is enough; temperatures improve reliability.

Can I still use my normal AC remote?

Yes, but during scheduled hours the device may revert power/temperature changes to

match the schedule. Other buttons (fan, swing) are left alone.

Why is the AC off even though I turned it on in the app?

You probably turned it on outside scheduled hours. The schedule forces it off at

those times. Adjust the schedule instead.

The presence light is off — is something broken?

No. It's only on when radar automation is enabled and a person is detected. Empty

room or radar off = light off. Normal.

Is solid magenta a fault?

No — it means "no internet, running locally." It's a safe fallback, not an error.

Does GSM/cellular do everything WiFi does?

Almost. The main exception: firmware updates over the air need WiFi.

I turned the AC off in the app and now the schedule won’t turn it back on. Is it broken?

No — that’s on purpose. A manual “off” is a firm hold: the device keeps the AC off and ignores the schedule and radar until you turn it on again. Just press Turn ON (or set a temperature) in the app to resume normal operation.

16. Do's and don'ts

Do

Point the device so it can "see" the AC, with nothing blocking it.

Aim the radar into the room, not at the doorway/corridor.

Empty the room before pressing Auto‑Calibrate.

Set the schedule in the app for how you actually live.

Don't

Don't expect manual remote temperature changes to stick during scheduled hours.

Don't put the radar facing a busy hallway (false triggers).

Don't worry about "No Internet" when connected to ECO_<device code> — that's normal.

Don't keep power‑cycling during a firmware update; let it finish.

17. One‑page quick reference (print this)

Setup hotspot: connect phone to ECO_<device code> / password password123

Open dashboard: browser → 192.168.4.1

Dashboard login: admin / admin

Developer Mode password: dev1234

Button: tap = enter Setup mode · hold 5 s = exit Setup mode

Status light:

Green flash every 5 s = healthy (WiFi) ✅

Magenta flash every 5 s = healthy (cellular) ✅

Solid magenta = no internet, running locally (safe)

Blinking cyan = Setup mode (connect to set it up)

Blinking blue = connecting to WiFi

Blinking amber/yellow = on WiFi, waiting for cloud

Fast red = error (power off/on)

Defaults: Normal 24 °C · Eco 26 °C · Eco delay 2 min · Off delay 5 min · Flap delay 10 s

Golden rules:

1. The schedule is the boss. Outside scheduled hours the AC stays off.

2. Empty room → Eco → Off, automatically. Someone returns → back on.

3. To "see" the AC and the room: keep the IR path clear and the radar aimed inside.

4. A manual OFF in the app stays off until you turn it back ON (it overrides the schedule until then).

*If you get truly stuck, note the status light colour/pattern and what you were doing,

and share that with your installer or support — it tells them almost everything.*