CLIMATEIQ V1

SMART AC CONTROLLER

Firmware and System Architecture Report

Stage: Production / Field-Deployed Firmware (v1.0.15)

Platform: ESP32 (Espressif) — Arduino / FreeRTOS

Classification: Confidential

Executive Summary

This document presents the complete firmware and system architecture for the ClimateIQ V1

smart air-conditioner controller. It serves as the primary technical reference for the engineering

team during development and maintenance, and for the client team for architectural review and

alignment.

The report covers every layer of the firmware stack, from hardware component selection and

requirements scope through to the firmware layer model, RTOS task and thread architecture, data

flow, power architecture, communication architecture, the operating-mode state machine, error

handling, and security. All decisions documented here represent the agreed architecture baseline

for the current production firmware. Tuning of parameters such as eco/off timers, radar detection

range, the "no-one" presence-hold window, schedule segments, and reconnect back-off intervals is

permitted during bring-up and field tuning, but no architectural or feature-level changes are

expected beyond what is defined in this document without a formal revision.

This document does not cover the cloud/back-end MQTT broker implementation, the customer mobile

or web front-end that issues commands, the AC unit's own internal electronics, or PCB mechanical

design. Those are intentionally out of scope for this report.

Any section marked with a Note or Assumption indicates a decision that was made based on

current project information and may be revisited in a future hardware or firmware revision.

1. Requirements Review and Scope Freeze

This section outlines the technical requirements, functional scope, and architectural

specifications for ClimateIQ V1, an ESP32-based smart controller that automates a standard

infrared-controlled split air conditioner based on room occupancy (mmWave presence radar) and a

7-day time schedule, with remote control and telemetry over the cloud.

The device is designed as a mains-powered, always-on retrofit controller. It learns the AC unit's

IR remote, then drives the unit through learned/universal IR commands while sensing room presence,

temperature and humidity. The primary objective of this baseline is reliable, unattended automation

that degrades gracefully when the network or clock is unavailable.

Critical Design Note

The radar hardware is the 14-gate HLK-LD2412 24 GHz mmWave module. The firmware drives it

through a local fork of the MyLD2410 driver (originally a 9-gate LD2410 library) adapted in

lib/LD2412/ to expose all 14 gates for per-gate energy reporting and range control. The upstream

9-gate library must not be re-introduced.

Hardware Architecture Requirements

ClimateIQ uses a single-board ESP32 design with externally wired sensors and an IR front-end. All

electronic components conform to the following scope.

Sensor & Component Bill of Materials (BOM)

The following components define the "locked" hardware scope for the production build:

| Component | Model / Spec | Function | Requirement Note |
| --- | --- | --- | --- |
| MCU | ESP32 DoIT DevKit v1 (ESP32-WROOM-32) | Main controller | Dual-core Xtensa LX6 @ 240 MHz, Wi-Fi + BT, FreeRTOS. |
| Presence Radar | HLK-LD2412 (24 GHz mmWave) | Occupancy / motion | 14 detection gates; UART @ 115200; per-gate moving/static energy. |
| Climate Sensor | HDC1080 (HDC1000-class) | Temperature + humidity | I²C @ 0x40; periodic ~10 s sampling. |
| IR Transmitter | IR LED via MOSFET driver | AC command output | 38 kHz; learned custom buttons + universal AC protocol fallback. |
| IR Receiver | IR demodulator (always-on) | Remote-press detection / learning | Detects foreign remote presses → manual-override reporting. |
| IR Power Switch | N-channel MOSFET (GPIO 27) | IR front-end power gate | Held active so the always-on receiver keeps listening. |
| Cellular Modem | SIM800-class / Quectel-style GSM | Connectivity fallback | UART (AT commands), shares the radar UART pins — see Note. |
| Status Indication | RGB LED (33/32/4) + status LED (2) | System / presence state | Common-anode RGB; per-state colour and blink patterns. |
| User Input | Momentary push button (GPIO 13) | AP entry / exit | Short press = enter AP config; long hold = exit AP. |

Pin Map (from include/Config.h)

| Function | GPIO | Function | GPIO |
| --- | --- | --- | --- |
| Config button | 13 | Status LED | 2 |
| IR MOSFET power | 27 | IR transmit | 18 |
| IR receive | 19 | RGB Red / Green / Blue | 33 / 32 / 4 |
| Radar UART RX / TX | 16 / 17 | GSM UART RX / TX | 16 / 17 |

Note — UART sharing: The radar and the GSM modem are mapped to the same UART pins (16/17). Consequently the radar and GSM cannot operate simultaneously; the device runs either the Wi-Fi + radar configuration or the GSM configuration, selected by the persisted switch_gsm_wifi flag. This is an accepted constraint of the current PCB revision.

Power System

Supply: Mains-powered (USB / DC adapter). The device is continuously powered and does

not run on a battery, so there is no sleep/duty-cycle power budget; the design favours

always-on responsiveness, a hardware/task watchdog, and fast network reconnection.

IR front-end: Powered through a MOSFET (GPIO 27) which is held active so the always-on IR

receiver can continuously detect manual remote presses.

Functional Scope (Software & Firmware)

The firmware scope is automation-first and resilience-first. The following behaviours

constitute the V1 delivery baseline.

Automation inputs

Radar presence — occupancy drives the AC on/eco/off state machine.

7-day schedule — per-day, up to 7 time segments, each with target temperature and optional

per-segment radar / eco-temp / eco-time / off-time overrides.

Cloud commands (MQTT) — temperature/power/radar/eco/schedule/OTA/dynamic-IR commands.

Manual remote — a foreign IR remote press is detected, reported, and reconciled.

Automation outputs

IR AC commands (learned per-temperature buttons, with a universal AC-protocol fallback).

MQTT telemetry: command ACKs, automation events, health alerts, periodic status.

Status/presence LED indication.

Operating modes / control sources (detailed in §8)

Schedule-driven control ("schedule is king" at boot).

Radar-driven energy-saving (normal → eco → off as the room empties).

Manual override (cloud or physical remote).

Offline failsafe (local radar control when the network/clock is unavailable).

AP configuration portal (Wi-Fi/GSM provisioning, IR learning, radar maintenance, local OTA).

Deliverables & Exclusions

Included in scope:

Functional firmware: radar + schedule automation, eco/off energy saving, enforce re-assertion.

Stable Wi-Fi/GSM MQTT transport with back-off reconnection and Last-Will.

IR learning (per-temperature buttons + protocol) and universal fallback.

Radar maintenance: calibration, factory reset, detection-range, presence-hold window.

Server-pull OTA with trial-boot rollback.

Local web dashboard / SoftAP configuration portal.

Explicitly excluded:

Cloud/broker back-end and command-issuing front-end applications.

AC unit internal electronics and mechanical/PCB design.

TLS-secured MQTT and authenticated OTA transport (see §10 — current baseline is plain transport).

2. High-Level System Architecture

ClimateIQ boots into a deliberate initialisation order (OTA rollback bookkeeping first, so a

crash-looping image can always be reverted), loads persisted settings from NVS, creates the shared

event queue / timers / IR mutex, then spawns the FreeRTOS tasks that perform all real work. The

main loop() runs at low priority and only feeds the watchdog, refreshes the LED, and executes any

deferred reboot.

POWER ON RESET
                                |
                                v
                 +------------------------------+
                 | OTAManager::init()           |  (FIRST: advance trial-boot
                 |  trial-boot / rollback book  |   counter, mark image valid)
                 +------------------------------+
                                |
                                v
                 +------------------------------+
                 | Hardware init                |
                 |  Indicator, irMutex,         |
                 |  IRManager, NVS (Preferences)|
                 |  SensorManager (radar+HDC)   |
                 |  WebDashboard routes         |
                 +------------------------------+
                                |
                                v
                 +------------------------------+
                 | Load state from NVS          |
                 |  temps, timers, radar flags, |
                 |  transport, schedule presence|
                 +------------------------------+
                                |
                                v
                 +------------------------------+
                 | Create queue + timers        |
                 |  automationQueue (10)        |
                 |  health(30s) enforce(3min)   |
                 |  eco / off (one-shot)        |
                 +------------------------------+
                                |
                                v
        +-------------------- Spawn FreeRTOS tasks --------------------+
        | SchedTask  AutoTask  SensorsTask  WebTask  OTATask  NetTask  |
        +-------------------------------------------------------------+
                                |
                                v
           Network boot (AP mode if no SSID) + task watchdog arm
                                |
                                v
            +---------------------------------------------+
            |  main loop() @ prio 1:                       |
            |   esp_task_wdt_reset() / LED / deferred boot |
            +---------------------------------------------+

At steady state the system is event-driven: hardware/sensor changes and timers post events to

the automation queue or notify the sensor task, and each task reacts within its responsibility

boundary. The supervisory loop never performs functional work.

3. Firmware Layer Architecture

The firmware is organised into clear responsibility layers. Application-level managers never touch

hardware registers directly; they call abstraction/service modules, which in turn use vendor

drivers and the platform layer.

1. Application Layer (product logic)

| Module | Responsible for |
| --- | --- |
| AutomationManager | The AC state machine (OFF / ON-NORMAL / ON-ECO); the single executeACCommand() choke point; eco/off/enforce event handling; presence-time accounting. |
| ScheduleManager | 7-day × 7-segment scheduler in NVS; minute-aligned evaluation; boot catch-up; offline failsafe; schedule-update ingest. |
| CommandProcessor | Inbound MQTT JSON command vocabulary → AC / schedule / radar / eco / OTA / dynamic-IR actions. |
| OTAManager | Server-pull firmware update with trial-boot rollback and phase reporting. |
| HealthManager | Periodic health checks (sensor staleness, low heap), reset-reason reporting. |

2. Service / Device-Abstraction Layer

| Module | Handles |
| --- | --- |
| SensorManager | Sole owner of the radar UART + IR receiver; radar/HDC polling; radar maintenance (calibrate/reset/range/no-one window); IR learn; presence eventing. |
| IRManager | AC IR transmit (learned button + universal fallback); always-on remote listener; learning; dynamic codes; all transmits serialised by irMutex. |
| NetworkManager | Thin transport abstraction; dispatches publish/telemetry to the active transport based on switch_gsm_wifi. |
| WebDashboard | SoftAP portal + local dashboard (config, IR learn, radar maintenance, dev diagnostics, local OTA). |
| Indicator | RGB/status/presence LED patterns; debounced config button. |

3. Transport Layer

| Module | Handles |
| --- | --- |
| WiFiManager | Wi-Fi STA + asynchronous MQTT (AsyncMqttClient); NTP sync; LWT; back-off reconnect; telemetry. |
| GSMManager | GSM/cellular MQTT via raw AT commands (QMTOPEN/QMTCONN/QMTPUB); RTC sync from the cell network. |

4. Driver / Library Layer

MyLD2410 (LD2412 fork) · Adafruit HDC1000 · IRremoteESP8266 · TinyGSM ·

AsyncMqttClient / PubSubClient · ArduinoJson.

5. Platform Layer (foundation)

Arduino-ESP32 core · FreeRTOS (tasks, queues, software timers, task notifications) · ESP-IDF

services (esp_task_wdt, esp_log, gettimeofday) · NVS via Preferences (namespace ir_data).

Visual Representation of the Layers

+---------------------------------------------------------------+
|                    CLIMATEIQ FIRMWARE                          |
|                                                               |
|  APPLICATION LAYER            SERVICE / ABSTRACTION LAYER      |
|  - AutomationManager          - SensorManager                 |
|  - ScheduleManager            - IRManager                     |
|  - CommandProcessor           - NetworkManager                |
|  - OTAManager                 - WebDashboard                  |
|  - HealthManager              - Indicator                     |
|                                                               |
|  TRANSPORT LAYER              DRIVER / LIBRARY LAYER           |
|  - WiFiManager (AsyncMqtt)    - MyLD2410 (LD2412 fork)         |
|  - GSMManager  (AT cmds)      - Adafruit HDC1000              |
|                               - IRremoteESP8266 / TinyGSM      |
|                               - AsyncMqttClient / ArduinoJson  |
|                                                               |
|  PLATFORM / RTOS LAYER        HARDWARE                         |
|  - Arduino-ESP32 core         - ESP32-WROOM-32                |
|  - FreeRTOS scheduler         - LD2412 radar / HDC1080        |
|  - ESP-IDF (wdt/log/time)     - IR TX+RX / MOSFET             |
|  - NVS (Preferences)          - GSM modem / RGB LED / button  |
+---------------------------------------------------------------+

4. RTOS Task and Thread Architecture

ClimateIQ runs on the Arduino-ESP32 FreeRTOS port. The design is event-driven: hardware and

sensor changes generate events that are processed asynchronously by dedicated tasks. Tasks are

isolated by responsibility and communicate through a message queue, FreeRTOS software timers, and

direct task notifications. The single shared state object (sysData) uses std::atomic fields for

any value touched by more than one task; all other fields follow a documented single-writer

convention.

Task-by-Task Definition

1. Schedule Task — `SchedTask`

Responsibility: Minute-aligned schedule evaluation; boot catch-up of the AC state; offline

failsafe (force radar control after 60 s offline with no clock).

Priority / Core / Stack: 4 / Core 1 / 4096 B.

Inputs: NVS schedule segments, system clock (NTP/GSM).

Outputs: executeACCommand(), radar-mode toggles, ACKs.

2. Automation Task — `AutoTask`

Responsibility: Drains automationQueue; drives the AC state machine; emits auto_event

telemetry; applies the "schedule is king" and "radar block outside schedule" policies.

Priority / Core / Stack: 4 / Core 1 / 4096 B.

Queue: automationQueue (10 × SystemEvent).

Event types: presence change, eco/off/enforce triggers, manual override.

3. Sensors Task — `SensorsTask`

Responsibility: Sole owner of the radar UART and IR receiver. Polls radar + HDC1080; runs the

always-on remote listener; performs radar maintenance on notification; reads boot range; accounts

presence time.

Priority / Core / Stack: 3 / Core 1 / 4096 B (handle captured for notifications).

Notification bits: (1<<1) calibrate · (1<<2) factory reset · (1<<3) set range ·

(1<<4) IR learn · (1<<5) set no-one window.

4. Web Task — `WebTask`

Responsibility: Services the web server / SoftAP portal and deferred AP exit.

Priority / Core / Stack: 1 / Core 0 / 4096 B.

5. OTA Task — `OTATask`

Responsibility: Idle until an ota_update command queues a job, then downloads + flashes the

inactive partition and reboots.

Priority / Core / Stack: 2 / Core 1 / 10240 B (covers the TLS/HTTPUpdate chain).

6. Network Task — `NetworkTask`

Responsibility: Services the active transport (Wi-Fi or GSM): reconnection back-off, MQTT

servicing, periodic telemetry.

Priority / Core / Stack: 1 / Core 1 / 8192 B.

Supervisory loop (`loop()`) — Priority 1, Core 1: watchdog reset, LED refresh, deferred reboot.

Software Timers

| Timer | Period | Type | Action |
| --- | --- | --- | --- |
| healthTimer | 30 s | periodic | Sensor staleness + low-heap checks (HealthManager). |
| enforceTimer | 3 min | periodic | Queue EVENT_ENFORCE_TRIGGER (re-assert AC state). |
| ecoTimer | configurable | one-shot | Queue EVENT_ECO_TRIGGER (room empty → eco). |
| offTimer | configurable | one-shot | Queue EVENT_OFF_TRIGGER (room empty → off). |

Priority Model

| Priority | Tasks |
| --- | --- |
| 4 (highest) | Schedule, Automation |
| 3 | Sensors |
| 2 | OTA |
| 1 (lowest) | Web, Network, main loop() |

Inter-Task Communication

Radar/HDC IRQ-poll          Timers (eco/off/enforce)     MQTT RX (Wi-Fi/GSM)
        |                              |                            |
        v                              v                            v
   SensorsTask  --EVENT_*-->   automationQueue  <--EVENT_*--   (CommandProcessor
        |                              |                         acts directly)
        |                              v
        |                        AutomationTask
        |                              |
        |                       executeACCommand()
        |                              |
        |                        IR (irMutex) ----> AC unit
        v
   xTaskNotify bits (web -> sensor task)        NetworkManager::publish* -> MQTT

Queue: automationQueue — the single event channel into the automation state machine.

Mutex: irMutex — serialises every IR transmit across tasks.

Notifications: xTaskNotify() bitmask wakes the sensor task for radar/IR maintenance (the

web/network tasks never touch the radar UART directly).

Shared atomics: presence, radar mode, range/no-one mirrors, AC state, schedule flags, and

presence accumulator live in sysData.

5. Data Flow Architecture

This section describes how data moves through ClimateIQ. In general: sensors and the cloud generate

events, the automation/schedule logic decides the AC action, IR drives the unit, and telemetry is

published back to the broker.

Sensor Data Path (presence → AC)

LD2412 radar --UART--> SensorManager::poll()
   -> debounce + flap-delay suppression
   -> cachedPresence (atomic)
   -> EVENT_PRESENCE_CHANGED --> automationQueue
        -> AutomationTask: ON-NORMAL / arm eco+off timers
        -> executeACCommand() --irMutex--> IR --> AC unit

HDC1080: sampled ~every 10 s on the sensor task; temperature/humidity stored for telemetry;

NaN reads trigger I²C re-init after a threshold.

Radar per-gate energy: moving/static energy for all valid gates is mirrored into sysData

for the dashboard developer feed (diagnostic only).

Command Data Path (cloud → device)

Broker --MQTT--> WiFiManager/GSMManager RX
   -> ArduinoJson deserialize
   -> CommandProcessor::processJSON()
        temperature_control | power_control | radar_control
        eco/teco/toff | ota_update | dynamic IR (protocol/state/code)
   -> AutomationManager / ScheduleManager / IRManager / OTAManager
   -> persist to NVS + publishACK()

Status / Telemetry Data Path (device → cloud)

Any task -> NetworkManager::{publishACK | sendAutomationEvent | publishHealthAlert}
   -> active transport (Wi-Fi async MQTT, or GSM AT QMTPUB)
   -> broker
WiFiManager/GSMManager loop() also publishes periodic telemetry (~10 s).

auto_event codes: 1000 normal-on · 2000 eco · 3000 off · 4000 manual override.

Schedule Data Path (clock → AC)

NTP (Wi-Fi) / cell RTC (GSM) -> system clock
   -> SchedTask (1 s poll, acts once per minute)
   -> findSegment() in NVS -> apply temp/radar/eco/off -> IR (if state changes)

Local Path (web/AP → radar/IR, no cloud)

SoftAP dashboard -> WebDashboard route -> SensorManager::requestX()
   -> xTaskNotify(sensorsTaskHandle, bit) -> SensorsTask applies over UART
   -> result mirrored back into sysData -> dashboard polls /devdata

6. Power Architecture

ClimateIQ is a mains-powered, always-on controller; unlike battery wearables there is no

deep-sleep/duty-cycle power budget. The power architecture therefore focuses on **clean power

domains, IR front-end gating, and uptime resilience** rather than energy harvesting.

DC / USB Adapter (5V)
                |
                v
        +----------------+        +------------------+
        | On-board reg.  |------->| ESP32 (3V3 rail) |
        | 5V -> 3V3      |        +------------------+
        +----------------+              |        |
                |                       |        +--> Radar (LD2412, UART)
                |                       +-----------> HDC1080 (I²C)
                v                       +-----------> RGB / status LEDs
        +----------------+              +-----------> GSM modem (when fitted)
        | IR MOSFET gate |
        | (GPIO 27, held |---> IR TX LED + always-on IR RX
        |  active)       |
        +----------------+

Design points

No sleep states: the device stays awake to keep MQTT, the radar stream, and the always-on IR

receiver live; resilience is provided by the watchdog and reconnection logic, not power gating.

IR front-end power: controlled by the MOSFET on GPIO 27, held active so the receiver keeps

listening for manual remote presses (which drive the manual-override path).

Reconnection over back-off-to-sleep: because the device is mains-powered, the Wi-Fi/MQTT

back-off is capped at 60 s (MAX_BACKOFF_MS) — it keeps retrying frequently rather than

backing off for minutes.

Assumption: Power-rail regulation and protection are handled on the carrier PCB. This firmware assumes a stable 3V3 supply and does not implement battery-management or brown-out mitigation beyond the ESP32's built-in brown-out detector and the task watchdog.

7. Communication Architecture

External Communication

Transport selection. NetworkManager dispatches all publish/telemetry calls to the active

transport based on switch_gsm_wifi (true = Wi-Fi, false = GSM). Both transports mirror the same

publish API (publishACK, sendAutomationEvent, publishHealthAlert).

| Interface | Stack | Role |
| --- | --- | --- |
| Wi-Fi + MQTT | AsyncMqttClient over Wi-Fi STA | Primary transport; LWT, subscribe, telemetry, async RX → CommandProcessor. |
| GSM + MQTT | Raw AT commands (QMTOPEN/QMTCONN/QMTPUB) | Fallback transport; PDP bring-up, RTC sync from the cell network. |
| NTP | pool.ntp.org + Google + Cloudflare | Wall-clock sync for the schedule (multi-server for filtered networks). |
| OTA (HTTP/S) | HTTPUpdate pull | Firmware download from the deployment server. |

MQTT broker: 107.172.137.233:1883 (plaintext). Device identity is the Wi-Fi STA MAC.

Inbound command vocabulary (CommandProcessor):

temperature_control — schedule update (with segments) or setpoint / ir mapping.

power_control — power_status on/off.

radar_control — enable/disable radar automation (manual override).

eco / teco / toff — eco setpoint and eco/off delays.

ota_update — url, version, optional force.

Dynamic IR — top-level protocol with state[] or code.

Internal Communication Interfaces

| Bus | Used for |
| --- | --- |
| UART (Serial1, 16/17) | LD2412 radar config + data stream (or the GSM modem — mutually exclusive). |
| I²C | HDC1080 temperature/humidity (addr 0x40). |
| IR (38 kHz) | AC command transmit (GPIO 18) + always-on receive (GPIO 19). |
| GPIO | RGB/status LED, config button, IR MOSFET gate. |
| NVS (flash) | Persisted settings: temps, timers, radar flags, schedule, IR codes, OTA bookkeeping. |

+------------------+
   LD2412 / GSM ----| UART (16/17)     |
   HDC1080  -- I2C--|                  |---- MQTT ---- Wi-Fi / GSM ---- Broker
   IR RX/TX -- 38k--|   ESP32 CORE     |---- NTP / cell RTC --------- Time
   LED/Btn -- GPIO--|                  |---- HTTP(S) ---------------- OTA server
   MOSFET  -- GPIO--|                  |
                    +------------------+

8. Mode State Machine Design

ClimateIQ runs two cooperating state models: the AC automation state machine (what the AC is

doing) and the system/connectivity state (what the device is doing on the network, used for LED

indication). On top of these sit the control-source policies that decide who may drive the AC.

AC Automation State Machine (AutoState)

room occupied (radar)
       AUTO_OFF ----------------------------> AUTO_ON_NORMAL
          ^  ^                                    |   ^
          |  |  off countdown elapsed             |   | re-occupied
          |  +------------------ AUTO_ON_ECO <----+   |
          |       eco countdown elapsed   |           |
          |                               +-----------+
          +-- schedule end / outside segment / cloud OFF

AUTO_OFF — AC off.

AUTO_ON_NORMAL — AC on at the scheduled/normal setpoint (occupant present).

AUTO_ON_ECO — AC at the raised eco setpoint (room empty for TEcoTime); after TOffTime the

room-empty timer drives it to AUTO_OFF.

Every transition is applied through executeACCommand(), which sends IR (learned button → universal

fallback), resets the enforce timer, opens the radar flap-delay blind spot on an OFF, flashes

the indicator, and publishes an ACK.

Control-Source Policy (who may drive the AC)

Schedule is king: at boot, radar may not drive the AC until the schedule has made its first

authoritative decision (scheduleBootDone). The offline failsafe is the sole exception.

Radar block outside schedule: when a schedule exists and the clock is valid but the current

time is outside all segments, radar events are dropped (unless the "radar outside" policy opts in).

Manual override: a cloud radar_control command or a detected physical remote press overrides

and is reconciled (the manual-override path re-asserts the intended state immediately).

Enforce re-assertion: every 3 minutes the intended AC state is re-sent to correct a

missed/garbled IR frame (suppressible per-feature via the backend toggle).

System / Connectivity State (SystemState, drives the LED)

SYS_BOOTING -> SYS_AP_MODE (no SSID / config)
            -> SYS_WIFI_CONN -> SYS_WIFI_OK  (Wi-Fi + MQTT up)
                             -> SYS_MQTT_DOWN (Wi-Fi up, broker down)
            -> SYS_GSM_CONN  -> SYS_GSM_OK
            -> SYS_ERROR
            -> (offline failsafe: magenta override)

Offline Failsafe

If the broker is unreachable and NTP never synced, after 60 s the schedule task forces radar control

so the room is still automated locally (magenta LED). When the clock later syncs, the schedule

re-asserts authority ("schedule is king") and the failsafe clears.

9. Error Handling Architecture

1. Overview

ClimateIQ implements a layered fault-detection and recovery strategy designed for an unattended,

always-on appliance: Detect → Recover → Report, with a hardware/task watchdog as the final

backstop and OTA trial-boot rollback protecting against bad firmware.

Detect            Recover              Report
   ------            -------              ------
   HealthTimer  -->  re-init bus     -->  MQTT health alert
   staleness         restart link         (publishHealthAlert)
   WDT / brownout    safe reboot          reset-reason on boot
   OTA trial boot    auto-rollback        OTA phase events

2. Error Categories

Critical (reboot / rollback)

MCU crash / hard fault, task watchdog timeout (120 s), brown-out.

Crash-looping new firmware after OTA → automatic rollback to the previous partition.

Major (subsystem recovery)

Radar UART stall / "frozen radar" (no frame for RADAR_STALE_MS = 30 s) → radar link re-init.

HDC1080 repeated NaN reads (≥ HDC_REINIT_THRESHOLD) → I²C re-init.

MQTT/Wi-Fi/GSM disconnection → back-off reconnect (cap 60 s).

Minor (filter / ignore)

IR repeat frames / noise / our own transmissions → filtered by the remote listener.

Radar false motion from the just-stopped AC airflow → suppressed by the flap-delay window.

Transient telemetry publish failures → retried on the next interval.

3. Recovery Mechanisms

| Domain | Mechanism |
| --- | --- |
| Radar | Staleness detection → attemptRadarRecovery() (UART re-begin + enhanced mode); config-mode resync before maintenance; BT lockout to prevent external BLE hijack. |
| Climate sensor | attemptHDCRecovery() (Wire re-init at 0x40) after repeated NaN reads. |
| Network | Exponential back-off reconnection (capped); MQTT LWT for clean "offline" signalling; periodic RTC resync. |
| System | Task watchdog (esp_task_wdt, 120 s) → reset; deferred-reboot mechanism; reset-reason reported on next boot. |
| Firmware | OTA trial-boot: a new image is committed only after MQTT reconnects; otherwise it auto-reverts after OTA_MAX_TRIAL_BOOTS (3) boot-loops (optional time-based revert). |
| Automation | Offline failsafe hands control to radar; enforce timer re-asserts intended AC state every 3 min. |

4. Fault Reporting

MQTT health alerts (publishHealthAlert) — event + detail.

OTA phase events — downloading / success / failed / rolled_back / skipped /

aborted (MQTT is intentionally down during the flash, so progress is reported as phases).

Serial logs — per-module ESP-IDF ESP_LOGx TAGs at INFO for app code, ERROR for libraries.

Reset reason — decoded and reported at boot (HealthManager::getResetReason).

10. Security Architecture

The current production baseline favours field-serviceability; the following describes the

implemented model and its known gaps (carried into §11).

Local Access (SoftAP / Web Dashboard)

SoftAP protected by a WPA2 passphrase (AP_PASSWORD).

Dashboard behind HTTP basic auth (WWW_USERNAME / WWW_PASSWORD); a separate

developer/diagnostic password (DEV_PASSWORD) gates the dev feed.

Note: Default credentials are shipped in Config.h. These must be changed/rotated per deployment; they are a known hardening item (see §11).

Cloud Transport (MQTT)

MQTT runs on plaintext port 1883 with a per-device client id (Wi-Fi MAC) and a Last-Will

message. There is no TLS or broker-side mutual authentication in this baseline.

Firmware Updates (OTA)

Integrity: the ESP32 ROM bootloader verifies the image SHA-256, and the trial-boot mechanism

guarantees a bad image is reverted.

Transport: the deployment server is plain HTTP, so setInsecure() is used. A root-CA pin

(OTA_ROOT_CA) is supported for HTTPS but is empty in this baseline — so the channel is not

protected against an active MITM serving a validly-signed malicious binary.

Radar Hijack Protection

The radar's Bluetooth is disabled and the setting persisted in the radar's own flash, so no

external BLE app can seize the radar's config mode and freeze the UART stream. A compile-time

RADAR_ENABLE_BT toggle re-enables BT only for bench testing.

11. Design Assumptions & Risk Analysis

1. Design Assumptions

Hardware

ESP32-WROOM-32 (DoIT DevKit v1) is the final MCU.

HLK-LD2412 (14-gate) is the presence sensor, driven via the local MyLD2410 fork.

HDC1080 is the climate sensor (I²C 0x40).

The device is mains-powered and continuously online.

Radar and GSM share UART pins 16/17 (mutually exclusive transports).

Firmware

Arduino-ESP32 + FreeRTOS is the base platform; NVS (Preferences, namespace ir_data) persists

all settings.

The schedule depends on a valid wall clock (NTP or cell RTC); the offline failsafe covers the gap.

IR control assumes the target AC is learnable, with a universal protocol as fallback.

Behaviour

The schedule is the authoritative baseline; radar provides energy-saving within it.

The cloud back-end and command front-end exist and speak the documented MQTT vocabulary.

2. Risk Analysis

2.1 MQTT plaintext + default credentials (HIGH)

Risk: Port 1883 is unencrypted; default web/AP credentials ship in firmware.

Impact: Eavesdropping / command injection / unauthorised local config.

Mitigation: Move to TLS MQTT and per-device credentials; force credential rotation on

provisioning; restrict the dev feed.

2.2 OTA over plain HTTP (HIGH)

Risk: setInsecure() with an empty root CA; integrity relies on ROM SHA-256 only.

Impact: An active MITM could serve a validly-signed malicious binary.

Mitigation: Serve OTA over HTTPS with a pinned OTA_ROOT_CA; sign release manifests.

2.3 Radar / GSM UART pin conflict (MEDIUM-HIGH)

Risk: Radar and GSM cannot run together (shared pins 16/17).

Impact: A GSM-fallback unit loses presence automation; a Wi-Fi unit cannot use GSM.

Mitigation: Treat as a deployment-time SKU choice; revisit pin allocation in the next PCB rev.

2.4 Radar stall / "frozen radar" (MEDIUM)

Risk: External BLE config or a desynced config state machine freezes the UART stream.

Impact: Presence automation stops.

Mitigation: BT lockout (persisted), 30 s staleness watchdog + UART recovery, config resync

before every maintenance operation. (Implemented.)

2.5 Clock dependency for scheduling (MEDIUM)

Risk: NTP blocked/slow on filtered networks; no clock → no schedule.

Impact: Schedule cannot evaluate.

Mitigation: Multi-server NTP, cell RTC on GSM, and the 60 s offline failsafe handing control to

radar. (Implemented.)

2.6 IR open-loop control (MEDIUM)

Risk: IR is fire-and-forget; the AC may miss a frame or be changed by a physical remote.

Impact: Device belief and AC reality diverge.

Mitigation: 3-minute enforce re-assertion, manual-remote detection + reconciliation, double-blast

custom buttons. (Implemented.)

2.7 Single shared NVS namespace (LOW)

Risk: All settings share one Preferences namespace and the global handle.

Impact: Key collisions / wear if extended carelessly.

Mitigation: Maintain a documented key registry; consider namespacing by subsystem.

12. Architecture Review & Sign-Off

This architecture document represents the agreed baseline for ClimateIQ V1 firmware v1.0.15. The

modules, task model, data flows, and policies described here are implemented in the current codebase.

Tuning of parameters (eco/off timers, radar range and presence-hold window, schedule segments,

reconnect back-off) is permitted without a document revision; structural or feature-level changes

require an update to this document and re-approval.

Open Items Carried Forward

| # | Item | Owner | Target Phase |
| --- | --- | --- | --- |
| 1 | TLS MQTT + per-device credentials | Firmware / Cloud | Next security revision |
| 2 | HTTPS OTA with pinned root CA | Firmware / DevOps | Next security revision |
| 3 | Resolve radar/GSM UART pin sharing | Hardware | Next PCB revision |
| 4 | Rotate default web/AP credentials per unit | Provisioning | Manufacturing |
| 5 | NVS key registry / namespacing | Firmware | Maintenance |

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Firmware Lead |  |  |  |
| Hardware Lead |  |  |  |
| Project Manager |  |  |  |
| Client Representative |  |  |  |

ClimateIQ V1 — Firmware & System Architecture Report — Confidential